LAB - IoT

Objetivos

- Una breve revisión sobre IoT

- implementar nuestro primer IoT

- johnny-five framework

- Ios integration IoT

Que IoT ?

El Internet of Things es el siguiente gran paso de la industria tecnológica Muy resumidamente, el Internet of Things es un concepto que se basa en la interconexión de cualquier producto con cualquier otro de su alrededor.

Install Johnny-five on Arduino

Para Instalar Johnny-five ejecutar el siguiente comando:

$ npm install johnny-five

If the above didn't work as expected, make sure that StandardFirmata is installed on the board:

- Download Arduino IDE

- Download NodeJs: https://nodejs.org/es/download/

- Plug in your Arduino or Arduino compatible microcontroller via USB

- Open the Arduino IDE, select: File > Examples > Firmata > StandardFirmata

- Click the "Upload" button.

- If the upload was successful, the board is now prepared and you can close the Arduino IDE.

Upload Firmata standart

https://github.com/rwaldron/johnny-five/wiki/Getting-Started#trouble-shooting

## Preparar el Servidor IoT

Instalar johnny-five : http://johnny-five.io/

- Install Node.js (Prefer 4.2.1 LTS).

- Setup your board.

- Run: npm install johnny-five

Run Hello world

create a file example.js and put the following code

var five = require("johnny-five");
var board = new five.Board();

board.on("ready", function() {
  // Create an Led on pin 13
  var led = new five.Led(13);
  // Blink every half second
  led.blink(500);
});

Run

node example.js

Correr Rest/API con johnny-five

Descargar arduinoControl.zip y correr el siguiente comando

$ npm install

$ npm start

Prender el LED

Apagar el LED

Crear la Interfaz

Create new Xcode project Single View and add Cocoa pods project:

- pod init

- Open Podfile and edit with following information:

pod 'Alamofire', '~> 4.5'

- uncoment #platform :ios, '9.0' of podfile

Configure insecure No HTTPS end point

<key>NSAppTransportSecurity</key>

<dict> <!--Include to allow all connections (DANGER)-->    <key>NSAllowsArbitraryLoads</key>

<true/>

</dict>

Conextar el UIswitch con el código

//

//  ViewController.swift

//  LEDONOFF

//

//  Created by jhonny on 7/26/16.

//  Copyright © 2016 Viva. All rights reserved.

//

import UIKit

import Alamofire

class ViewController: UIViewController {

@IBOutlet weak var ledswitch: UISwitch!

override func viewDidLoad() {

super.viewDidLoad()

ledswitch.on = false

// Do any additional setup after loading the view, typically from a nib.

}

override func didReceiveMemoryWarning() {

super.didReceiveMemoryWarning()

// Dispose of any resources that can be recreated.

}

@IBAction func ledAction(sender: AnyObject) {

if(ledswitch.on){

print("on switch")

ledControl("on")

}else{

print("off switch")

ledControl("off")

}

}

func ledControl(status:String) {

Alamofire.request("http://localhost:3000/arduino/led/\(status)", method: .get).responseJSON { response in

// TODO implement

}

}

}